# Control Systems – Documentation (Docx‑style)

---

## 1. Overview

A **control system** manages, commands, directs, or regulates the behavior of other devices or systems using control loops.  It can range from a single home heating controller (thermostat → boiler) to large industrial control systems that manage processes or machines.  Control systems are designed via the *control engineering* process.

---

## 2. Core Concepts

| Concept | Description |
|---------|-------------|
| **Process Variable (PV)** | The actual measured value of the system being controlled. |
| **Setpoint (SP)** | Desired target value for the PV. |
| **Control Signal** | Output from the controller that drives actuators to bring PV toward SP. |

---

## 3. Control Architectures

### 3.1 Open‑Loop vs Closed‑Loop Control
- **Open‑loop**: No feedback from the plant; controller acts solely on a preset command.  
- **Closed‑loop (feedback)**: Continuously compares PV with SP and adjusts the control signal accordingly.

### 3.2 Feedback Control Systems
- Utilizes a *feedback controller* (e.g., PID, LQR) to automatically adjust the process.  
- The controller evaluates the error **e(t) = SP – PV** and applies a corrective action.

### 3.3 Logic Control
- **Historical**: Electrical relays & cam timers with ladder logic.  
- **Modern**: Microcontrollers or programmable logic controllers (PLCs).  
- Programming styles: Ladder Diagrams, Sequential Function Charts (SFC), Statement Lists.

---

## 4. Control Strategies

| Strategy | Typical Use | Example |
|----------|-------------|---------|
| **On‑Off Control** | Simple threshold-based systems. | Domestic thermostat, compressor pressure switch |
| **Linear Control** | Systems that can be approximated by linear models. | PID controllers for temperature, speed |
| **Fuzzy Logic** | Non‑linear, uncertain systems where rules are expressed in natural language. | Furnace temperature regulation |

---

### 4.1 On‑Off Control
- **Mechanism**: Switches abruptly between two states (on/off).  
- **Implementation**: Bi‑metallic thermostat, pressure switch.  
- **Pros/Cons**: Cheap and effective for many applications; may cause wear due to frequent switching.

### 4.2 Linear Control
- **Approach**: Uses linear models and controllers (PID, LQR).  
- **Benefit**: Well‑understood theory; efficient for many industrial processes.

### 4.3 Fuzzy Logic
- **Concept**: Allows partial truth values; rules are linguistic (“If temperature is high…”).  
- **Process**:
  1. *Fuzzification*: Convert crisp measurements to fuzzy sets.  
  2. *Rule Evaluation*: Apply fuzzy logic operations.  
  3. *Defuzzification*: Convert fuzzy result back to a crisp control action.  
- **Use‑Case**: Scalable for complex systems where conventional linear design is cumbersome.

---

## 5. Physical Implementation

| Type | Typical Hardware | Notes |
|------|------------------|-------|
| **Compact Controllers** | Dedicated microcontroller boards, single‑purpose PLCs | Ideal for small machines or devices |
| **Distributed Control Systems (DCS)** | Networked PLCs, RTUs, HMI panels | Used in large plants; high reliability |
| **Frameworks** | BREAD (Broadly Reconfigurable and Expandable Automation Device) | Open‑source hardware for modular, scalable control |

---

## 6. Software & Tools

| Tool | Description |
|------|-------------|
| **Python Control Systems Library** | Simulate/control loops, Kalman filter, LQG |
| **MATLAB Control System Toolbox** | Design, analysis, and simulation |
| **Python Control (PyConSys)** | AI‑assisted PID tuning, loop simulation |

---

## 7. See Also

- Control Engineering  
- PLC Programming Languages (Ladder, SFC)  
- Feedback Loop Theory  

---

## 8. References

*(Add references here as needed, e.g., textbooks, IEEE papers.)*

---

## 9. External Links

- [Python Control System](https://python-control.readthedocs.io) – Simulate and design control loops with Python.  
- [MATLAB Control System Toolbox](https://www.mathworks.com/products/control.html) – Design and analyze control systems.  
- [BREAD](https://bread-project.org) – Open‑source reconfigurable automation hardware.  

---